mcOCR v. 3.00

USER’S GUIDE

2019-06-13

Published by:

Demmery Software

#24 – 612 MacLaren Street

Ottawa, ON   K1R 5K9

CANADA

TABLE OF CONTENTS

INTRODUCTION

mcOCR is a program with a very specific purpose: to capture weather data from LCD display panels. Data can be extracted for up to 4 locations, 16 values, and 10 characters per value.

It is important that you be up on this terminology, as it is used throughout this guide.

Locations are data elements grouped together, such as you would see on an inside/outside weather display. You can specify up to four locations in this way.

Values are individual data elements: temperature, relative humidity, dew point, and so on.

Characters are the individual characters that make up a value. Characters are scanned, and their values are placed together. If a character scan fails, then the value is assigned nil.

Segments are the individual segments that make up a character. WxOCR scans these, applying numeric values to each segment. It then consults a table of scan-values-to-real-values to come up with the actual value scanned.

mcOCR is designed for performance: a well set-up instance of the program can detect data when the

source image is practically unreadable. Further, special-case directives allow you to can:

- Straight Data (e.g. temperature);

- Differential Data (where a value is constantly increasing, such as from a rain gauge);

- Darkest (weighted) (where only one of up to sixteen segments can be lit, such as from a wind-direction indicator).

With mcOCR v. 3.00, it’s easier than ever to maintain your .ini file—every possible tag has an equivalent in the Settings screens. You will never need to hand-edit your .ini file again!

mcOCR gives you tremendous control over your locations, values, even individual characters and segments. Characters can be autocalculated according to an outside set of coordinates; or, you can specify the coordinates of individual segments.

Stats Display

The stats display covers one location at a time. All possible weather data are displayed; if a corresponding data element has not been defined, the field will simply be left blank. You can select which of your locations to display by selecting it in the combo box. Items are displayed based on their specifications: if Mul10 or Mul100 is selected, they will be displayed as x.x or x.xx, respectively. If they are specified just to be integers, then they will be displayed as such.

mcOCR writes output to files based on a pattern that you provide. Even the output format of these files is configurable. Where the files ultimately go is up to you, but they are output to the Outputs folder.

mcOCR provides error-correction, for character misreads. It’s not perfect, but it’s better than nothing, and output files in which error correction has taken place can be denoted by -EC being appended to the filename, though a user-configurable switch.

And what about this error-correction? mcOCR for each character keeps a list of valid scan codes, as well as codes for similar, but misformed, characters. Often a misread can be tracked fairly reliably to a particular character. mcOCR will do this—and it’s controlled by you.

mcOCR has a wealth of other features, including unit conversions, copying from existing values, arithmetic at various stages in the scanning process. It can even calculate values based on other values. All of this is under your control.

THE SCANNING PROCESS:

It is important to understand how mcOCR goes about scanning.

Scanning begins on the segment level. For each segment defined for a character, mcOCR scans first a control area, then all the pixels in a segment, coming up with a brightness value. The control value is divided by the threshold value, then multiplied by 100. If the segment brightness value is lower than the resulting control brightness value, then the segment is deemed to have been illuminated.

Segments are assigned unique numeric values. During scanning a character, the values are added up for all segments which are illuminated; the result is the Scan Code. Next, mcOCR consults a table of scan codes and their corresponding character values. Allowing for error-correction, there can be multiple scan codes for each value.

If, at any point in the scan, a character cannot be read, then the value is set to nil, and no further processing takes place.

Otherwise, mcOCR takes the resulting character string, converts it to a numeric value, manipulates it as specific in the .ini file, and stores it away for output. At the end of a scan (not in Hold Mode), the values are written to an output file, one per location.

You are limited to the following:

- 4 Locations

- 16 Values/Location

- 10 Characters/Value

- 16 Segments/Character

In addition, mcOCR has provision for input of deduced sky conditions, and will calculate sunshine in that case.

mcOCR.ini FILE

The wxocr.ini file controls the program, and it is quite complex.

In format, it is similar to an XML file, in that it represents a hierarchical collection of Locations, Values, Characters, and Segments. Indenting is encouraged, for readability.

Here are the declarations possible in wxocr.ini:

Global Declarations:

SaveGood:		Save images that have been successfully scanned

SaveBad:		Save images with errors

Gslant integer: 	Set the Global slant value to the integer value provided

Location Declarations:

Location string: 	Begin a location definition; name is in string.

OutputDef: 		A string that defines the output. (IN DEPTH)

ShowEC:		Show when error-correction has been used (by appending -EC to filename)

LSlant integer:	All values in the location should have slant value assigned.

Value Declarations:

Value string:	Define a value named (string).

Measure:	This value will be scanned from the image

Mul10, Mul100:	The value detected should be multiplied by (10, 100) prior to output.

MinValid integer, MaxValid integer:	Defines acceptable minimum and maximum values. If the scanned value is outside the range, it will be reset to nil.

Floor integer, Ceiling integer:	Defines ceiling and floor values for a value. Scanned values outside this range will be corrected to the ceiling or floor value.

Track: 	Track this value; if it is in error, then the image output will be marked bad.

Differential:	This value is obtained by subtracting the previous value (for example, a rain gauge which simply records the total precipitation fallen since it was turned on).

PreAdd real, PreSub real, PreMul real, PreDiv real: 	Specifies arithmetic to be performed on the value immediately after scanning. Applies only to scannable values.

Add real, Sub real, Mul real, Div real: 	Specifies arithmetic to be performed after converting a value. Applies only to values with conversions.

PostAdd real, PostSub real, PostMul real, PostDiv real:	Specifies arithmetic to be performed after calculating a value. Applies only to values with calculations.

Copy string:	Copy from another value. The value must be named correctly.

Convert string:	Convert from one unit scale to another. Possible conversions include:

IntoMb: inches of mercury to millibars

MbtoIn: millibars to inches of mercury

FtoC: Fahrenheit to Celsius

CtoF: Celsius to Fahrenheit

MphtoKmh: miles per hour to kilometres per hour

KmhtoMph: kilometres per hour to miles per hour

IntoCm: Inches to centimetres

CmtoIn: Centimetres to Inches

IntoMm: Inches to millimetres

MmtoIn: millimetres to inches

+CalcHumidex(temperaturevaluename, dpvaluename):	Calculates the humidex, based on the temperature value and dew point values supplied.

+CalcDewPoint(temperaturevaluename, rhvaluename):	Calculates the dew point, based on the temperature and relative humidity values supplied.

+CalcWindChill(windvelocityvaluename, tempvaluename): Calculates the wind chill index, based on the temperature and wind velocity values supplied.

Character Declarations:

character: 		Defines a character

weighted:		All segments of the character will be scanned, and the darkest segment selected.

Thresh:		Sets the threshold value for the character. See separate section.

Control:		Sets a relative-coordinates control area. The following line should contain:

{xoffset},{yoffset},{size}

Some examples

Control
			  +6,+4,10

-2,+5,15

Control-Abs: 		Sets an absolute-coordinates control area. The following two lines should contain:

X1,Y1

X2,Y2

Example:
			Control-Abs

101,240

106, 247

AutoXY1		Set the upper-left bounds of an auto-calculated character.

AutoXY2		Set the lower-right bounds of an auto-calculated character.

Auto Small H		Auto-calculate for a small LCD character, horizontal

Auto Small V		Auto-calculate for a small LCD character, vertical

Auto LCD H		Auto-calculate for a large LCD character, horizontal

Auto LCD V		Auto-calculate for a large LCD character, vertical

WHAT’S ALL THIS ABOUT THRESHOLDS?

McOCR is all about thresholds. Each character and each segment can be assigned its own threshold value.

When mcOCR scans most characters, it first scans a “control area” nearby, and compares the scanned segments to it. If the segment values are lower than the control area value, then the segment is deemed to be illuminated.

Of course, the lighting in an image can be uneven; and the image quality varies. If your images show good contrast and crisp characters, you may only ever have to set nominal values for your characters; if, on the other hand, your image is of low quality, you may spend considerable time specifying each segment’s threshold value.

INSTALLING mcOCR

There’s really not much to installing mcOCR.

Place it in a folder where it makes sense to be. That could be in your /home folder; it could be in a special ‘weather’ folder that you’ve created.

UnZIP the downloaded file into the directory, and you will have a default installation. Note that there are a number of folders created: webcam, Outputs, and backup. The backup folder carries a number of test images, taken by the data-capture camera; and the default wxocr.ini file is configured to interpret those images.

To run mcOCR, cd into its folder, and enter the following command:

./mcocr

HOW IT WORKS:

mcOCR checks twelve times per minute for an incoming webcam image in the folder webcam (the location can be configured). When an image is detected, mcOCR loads it and performs the OCR analysis on it, displaying the results.

EQUIPMENT SETUP

You will need a decent webcam to use mcOCR. You will want a light source, and you will want to construct some sort of image-capture stage whose light level is very constant.

Basically, you will want to mount your LCD display so that it is standing, then mount the camera so that it has a good view. Adjust the lighting so that it is even throughout the frame. The better contrast, the more easily mcOCR can capture data.

Now, you’ll want a webcam-image-capture program, and a cron job to trip it off according to your interval (I use every five minutes). I suggest fswebcam, an old utility, but a good one. It allows you to scale your images and save more than one.

Please note that clarity—crispness, contrast—should be your overarching goal; the size of the image isn’t really important. So give your camera some distance to focus.

A narrow-aperture light source is best, as it provides stronger shadowing against the background and may help the readability of your image.

Here is how I have my cameras set up (please forgive the crudity of the image; if there’s one thing I’m no good at, it’s graphic design; and the image is not to scale):

Missing from the image: a large cardboard light-shield to block stray sunlight.

The output filenames should be:

The files should be transported to the mcocr/webcam folder, where mcOCR will process them.

THE MAIN SCREEN

The Main Screen of mcOCR displays the most-recent image, along with the data captured from it. As you can see, there are plenty of other controls on the screen.

[Reserved for image of main screen]

Onscreen Elements:

The Image View:

When an image is loaded, it is displayed here. All of the values will be plotted in a visual fashion; red segments indicate where the program detected illuminated segments; blue for not illuminated.

Sky-Condition buttons:

Certain sky ‘conditions’ are difficult for mcOCR to detect; therefore, a set of buttons is provided, allowing you temporarily to specify an additional sky condition. The length of time to be taken is specified in the Duration field.

Control Buttons:

There are several of these, including Hold, Reprocess, Reload, Exit, and Settings.

When you click on Hold, Hold Mode is automatically engaged. The button will change its caption to “Rel All”, and an additional button will appear: “Release One”.

- Release One writes the readings to disk and loads the next image.

- Release All releases the Hold

When you click the “Reprocess” button, the image is reprocessed with the latest settings. No additional information is written.

The Reload button reloads the wxocr.ini file from disk. You should do this immediately after manually editing the file.

Clicking on Exit will, of course, exit the program.

The Settings button will engage Hold mode automatically and take you to the Global Settings page (covered separately).

ADJUSTING mcOCR

Once you’ve got your program running, you’ll need to adjust the threshold values of your characters.

You can do this by eyeballing the image display:

- A character image that is missing segments needs a higher threshold value. Try incrementing by 5 at first, later by 2 and 1.

- A character image with false-positives needs a lower threshold value. Try decrementing by 5 at first, later by 2 and 1.

- A character with a mix of false-positives and missing segments will need to have individual SegThresh values. Go by the same rule: if it’s missing, increment the threshold value; otherwise increment.

I suggest changing your values by 5 at first, because otherwise you’re going to spend weeks adjusting the thing!

You can change threshold values for any object through the settings screens.